|  | AT SEA SHIP TO SHIP TRANSFER STS SUPERINTENDENT STANDING ORDER | Form No:; OPS-OFD-011. Rev.No.:; 3.0. Rev Date:; 20 Feb 2026. Approved by:; JS. Page:; 1 of 2. |
| --- | --- | --- |

STS transfer operations are under the advisory control of the STS Superintendent. Each Master remains responsible for the safety of their own ship, crew, cargo and equipment and should not permit safety to be prejudiced by the actions of others.

Environment

Inform Superintendent if:

Weather forecast indicates adverse conditions.

Wind speed is increasing unexpectedly or consistently gusting above 20kts or as directed by the STS Superintendent.

Local weather forecasts indicate an approaching deep low-pressure system or gusting winds.

There are electrical storms in the vicinity.

The vessel does not appear to be holding position.

Communications

During cargo operations, essential personnel on both ships must have reliable and common means of communication.

In case of communication breakdown on either ship, the emergency signal should be sounded, and all operations suspended immediately.

Operations should not resume until satisfactory communications are re-established.

Emergency Situations

The agreed Emergency signal must be clearly understood by personnel on both ships.

In an emergency, both vessels should immediately implement the appropriate contingency plan.

Examples of emergency situations requiring suspension of cargo operations and calling the STS Superintendent are:

• Accidental cargo release

• Gas accumulation on deck

• Any leakages at the manifold

• Onset of adverse weather conditions

• Electrical storms

• Ships emergency

• Safety infringements

State of readiness for an emergency:

The following arrangements should be made on both ships:

Main engines steering gear ready for immediate use.

Crew available and systems prepared to drain and disconnect hoses at short notice.

Oil spill containment equipment prepared and ready for use.

Mooring equipment ready for immediate use, and extra mooring lines ready at mooring stations as replacements in case of breakage.

Firefighting equipment ready for immediate use.

STS SUPERINTENDENT STANDING ORDER

|  | AT SEA SHIP TO SHIP TRANSFER STS SUPERINTENDENT STANDING ORDER | Form No:; OPS-OFD-011. Rev.No.:; 3.0. Rev Date:; 20 Feb 2026. Approved by:; JS. Page:; 2 of 2. |
| --- | --- | --- |

Hoses

Hoses and their securing arrangement should be inspected during cargo operations.

Hose connections are leaking.

There is excessive movement of the hoses or hose kinking.

If there is any doubt about the positioning of the hoses.

Moorings

Moorings should be inspected frequently and adjusted accordingly. Call STS Superintendent if:

Any moorings fall.

the other vessel does not appear to be tending its mooring.

Vessels are experiencing increased movement.

If there is any doubt about the condition of the moorings.

Fenders

Fenders must be inspected regularly during the cargo transfer operation. The fender moorings should be tended as required. Inform STS Superintendent if the following is observed:

Damage to fenders, fender moorings or associated equipment.

There is excessive movement of fenders.

If there is any doubt about the condition or position of the fenders.

Cargo Operations

Maintain hourly exchange of cargo rate and quantity transferred with another vessel. Inform the STS Superintendent if there is a significant difference in figures.

Ensure a crew member is always at the manifold.

Call STS Superintendent if there are any changes to or deviations from the cargo transfer plan.

STS Superintendent Specific Instructions

Ullamco molestias re

| Master's Name: Breanna Olson Ship's stamp: | SS/MV: Erich Cash Date (dd/mmm/yyyy): 18 Dec 2011 Time (HH:MM): 08:46 |
| --- | --- |

STS SUPERINTENDENT STANDING ORDER